SOURCE CONTROL SPECIFICATION

Part Number: E.SLQ004

Only the item described in this source control to meet the specification listed herein is approved by SILQ.  A substitute item shall not be used without prior approval.

# Purpose

## This document provides the specification for the 500 gallon waste tank assembly.

## The Waste Tank Assembly is to accommodate intermediate, temporary storage of permeate and aqueous waste.

# Waste Tank Specification

## Manufacturer Part Name:  Norwesco 500 Gallon X 30 Deg Cone Bottom Tank with Stand

## Manufacturer Part Number:  CB-500-W/STD-NH

## Tank Capacity: 500 gallons

## Dimensions (Diameter x Height): 49” x 87”

## Material: Polyethylene

## Connection: 2” Poly Bulkhead Tank Fitting (PBM Supply Item #TF200)

# Assembly

## Top Components

### Thick-Wall Through-Wall Pipe Fitting, UV-Resistant Polypropylene for Chemicals, Connector, 1 NPT Female

### PTFE Thread Sealant Tape

### CPVC Pipe Fitting for Hot Water, Bushing Reducing Adapter, 1 NPT Male x 1/2 NPT Female

### Super-Flow Barbed Fitting for Food and Beverage, Straight Adapter for 3/8" Tube ID x 1/2 NPT Male

## Bottom Components

### UV-Resistant Polypropylene Pipe Fitting for Chemicals, 90 Degree Elbow Adapter, 2 NPT Female x 2 NPT Male

### Thick-Wall Polypropylene Pipe Fitting for Chemicals, Straight Connector, 2 NPT Male, 6" Long

### Easy-Install Double-Union Polypropylene On/Off Valve for Water, T-Handle, Black, 2” NPT Female

### UV-Resistant Polypropylene Pipe Fitting for Chemicals, Bushing Reducing Adapter, 2 NPT Male x 1 NPT Female

### Super-Flow Barbed Fitting for Food and Beverage, 1" Tube ID x 1 NPT Male

## The following modifications and additions have been made to the waste tank

### Through Wall Pipe Fitting

#### Drill a hole suitable in diameter for attaching the through-wall pipe fitting.

#### Approximate location at the top of 500GAL – Waste Tank is demarked by a red asterisk in Attachment 1 – 500 Gallon Cone Bottom Tank and Stand drawing.

#### Insert Through-Wall pipe fitting and secure to tank.

### Waste Inlet Fittings

#### At top of tank, secure CPVC Bushing reducing adapter 1 NPTM x 1/2 NPTF to through-wall fitting. Secure 3/8” Barb x 1/2 NPTM fitting to bushing. 3/8” ID hose from permeate / aqueous waste line may now be installed.

### Waste Outlet Fittings

#### At bottom of tank (cone bottom), secure 90-degree elbow adapter. Secure 2” NPT male 6” straight connector. Secure Double-Union Polypropylene T-Handle On/Off Valve. Secure 2” NPTM x 1” NPTF Bushing reducing adapter to on/off valve. Secure 1” Barbed x 1” NPTM fitting to reducing adapter. Secure 1” dia. Hose to barbed fitting for WASTE OUT to waste diaphragm pump (E.SLQ003).

# Attachments

## Attachment 1 – 500 Gallon Cone Bottom Tank and Stand drawing

## Attachment 2 – 500 Gallon Cone Bottom Tank drawing

## Attachment 3 – Waste Tank Fitting Assemblies drawing