SOURCE CONTROL SPECIFICATION

Part Number: E.SLQ007

Only the item described in this source control to meet the specification listed herein is approved by SILQ.  A substitute item shall not be used without prior approval.

# Purpose

## This document provides the specification for the pneumatic drum stirrer.

# Supplier

## McMaster-Carr. 9630 Norwalk Blvd., Santa Fe Springs, CA 90670, Tel: (630) 833-0300. www.mcmaster.com

# Reference Part Number(s)

## Manufacturer Part Name:  Air-Powered Mixer for Drums

## Manufacturer Model Number:  35005K64

# Specification

## Horsepower: ½

## Mixer speed: 300 – 1750 rpm

## Propeller:

### Number of: 1

### Diameter: 3.75”

### Material: 316 Stainless steel

## Shaft:

### Length: 29”

### Diameter: 5/8”

### Material: 316 Stainless steel

## Air Connection:

### Pipe size: ¼

### Thread type: NPT

### Gender: Female

## Capacity: 55 gallon

## Power Source: Air

## RoHS: RoHS 3 (2015/863/EU) Compliant

## Overall height: 39-5/8”

## Weight: 29 lbs

# Drum Stirrer Holder

## 2” NPT Coupler welded to 2” square stock tube, welded to floor plate.

# Attachments

## None