SOURCE CONTROL SPECIFICATION

Part Number: E.SLQ011

Only the item described in this source control to meet the specification listed herein is approved by SILQ.  A substitute item shall not be used without prior approval.

# Purpose

## This document provides the specification for the large flow Peristaltic Pump.

# Supplier

## Any supplier that sells/distributes the specified product.

# Manufacturer

## U.S. Solid, 1200 E. 55th St., Cleveland, OH 44103, Tel: 800-243-5428.

# Reference Part Number(s)

## Manufacturer Part Name:  Peristaltic Pump M6-12L Industrial Tubing Pump Large Flow 0.69-12000 mL/min

## Manufacturer Model Number:  M6-12L

# Specification

## Flow rate range: 0.69 – 12,000 mL/min

## Speed range: 0.5 – 600 rpm

## Flow rate accuracy: < ±0.5%

## Outlet pressure: 0.3 MPa

## Housing material: 304 Stainless steel

## Diaphragm material: Santoprene

## Power supply: 110 – 220 VAC ±10% 50Hz/60Hz

## Power consumption: < 300W

## Dimensions (L x W x H): 9 51/64” x 12 13/32” x 12 9/16”

# Attachments

## U.S. Solid M6 Series Peristaltic Pump Manual