SOURCE CONTROL SPECIFICATION

Part Number: E.SLQ017

Only the item described in this source control to meet the specification listed herein is approved by SILQ.  A substitute item shall not be used without prior approval.

# Purpose

## This document provides the specification for the temperature and humidity monitor.

# Supplier

## Cole Parmer, 625 East Bunker Ct Vernon Hills, IL 60061, Phone number: 800-323-4340, website: www.coleparmer.com

# Manufacturer

## Traceable® Products, 12554 Galveston Rd., Suite B230, Webster, TX 77598, Tel: (281) 482-1714.

# Reference Part Number(s)

## Manufacturer Part Name:  Monitoring Traceable Hygrometer

## Manufacturer Model Number:  4040

# Specification

## Relative humidity range: 20 – 90%

## Relative humidity accuracy: ±5% RH mid-range; ±8% RH otherwise

## Relative humidity resolution: 1%

## Temperature range: 0 – 50℃ (32 – 122℉)

## Temperature accuracy: ±1℃

## Temperature resolution: 0.1°

## Power: One (1) AAA battery

# Attachments

## Traceable 4040 Instruction Manual