Design Input

Standard Operating Procedure

# Purpose

## The purpose of this standard operating procedure is to establish the process of defining user needs and design inputs for medical devices developed at SILQ to ensure design activities are in compliance with 21 CFR 820.30 and ISO 13485:2016.

## This procedure describes the general process to identify, document and approve user needs and product requirements for a given product.  In addition, this procedure details a system to resolve incomplete, ambiguous, or conflicting product requirements.

# Scope

## The following procedure applies to all design input activities during the development of medical device product lines at SILQ.

## In addition, all design input activities performed by approved subcontractors for or in support of design control projects at SILQ must be in compliance with the requirements defined in this procedure.

# Documents

## Reference Documents

### QM.SLQQM.SLQ001	Document Control

### QM.SLQQM.SLQ004	Design Control Program

### QM.SLQ007			Design Output

### QM.SLQ008			Design Review

### QM.SLQ009			Design Verification and Validation

### QM.SLQ013			Risk Analysis

## Associated Forms / Templates

### TMP1-QM.SLQ006	User Needs (Design Input) Template

### TMP2-QM.SLQ006	Product Requirements (Design Input) Template

# Definitions

## Abbreviations / Acronyms

### DCO:  Document Change Order

### DHF:  Design History File

### DMR:  Device Master Record

### QA:  Quality Assurance

### QMS:  Quality Management System

### SOP:  Standard Operating Procedure

### V&V:  Verification and Validation

## General Definitions

### Design History File (DHF): A compilation of records that describes the design history of a finished device.

### Design Input: The physical and performance requirements relating to the device and its intended use. These requirements establish a basis for performing subsequent design tasks, and focus on the needs of the users and patients.

### Design Output: The results of a design effort at each design phase and at the end of the total design effort.  The finished design output is the basis for the device master record.  The total finished design output consists of the device, its packaging and labeling, and the device master record.

### Design Review:  A documented, comprehensive, systematic examination of a design to evaluate the adequacy of the design requirements, to evaluate the capability of the design to meet these requirements, and to identify problems.

### Document Change Order (form):  The form used to add new or change existing documents in the document control system.  The required information is listed in the form to manage the document control process and record the required approvals.

### FileHold: Software based document management system used to electronically store controlled documents.

### Validation:  Confirmation, by examination and provision of objective evidence, that the particular product requirements (design inputs) for a specific intended use (user needs) can be consistently fulfilled.

### Verification:  Confirmation, by examination and provision of objective evidence, that specified requirements (design inputs) have been fulfilled by the product design (design outputs).

# Responsibilities

## It is the responsibility of Quality Assurance management to ensure this procedure and its referenced procedures are followed during design and development of the product line at SILQ.

## It is the responsibility of Manufacturing and R&D/Engineering management in conjunction with the assigned design team or group to collect and record design inputs in accordance with this procedure.

## It is the responsibility of SILQ personnel involved in a product’s development life cycle to provide or forward input to R&D/Engineering on product requirements as necessary.

# Procedure:  Defining User Needs

## The safe and effective design of a device is rooted in a clearly defined set of user needs and design inputs (also referred to as product requirements).  The definition of these requirements establishes the set of characteristics needed by the device to safely and properly function and perform.

## A defined set of user requirements are essential in qualifying the product design meets its intended use.

## User needs represent the initial set of top-level requirements that define the therapeutic, diagnostic, or monitoring capability required of the device(s).

## An indications for use (or intended use) statement is to be developed to establish the overall purpose of the device and its clinical mode of operation.  The indications for use statement is typically based on or influenced by previous indications for use statements for similar existing devices.  Regulatory Affairs is either to be given the responsibility or consulted with for the development of the indications statement.

## As part of defining user needs, the potential users of the device are to be identified and characterized.  This includes both the patient and healthcare provider(s) that will use, interact and/or be affected by the device.

## After identifying the user characteristics, a general outline of the clinical procedure or use is to be defined.

## In addition, marketing needs or requirements by strategic partners can also be defined with the user needs to establish device characteristics that are essential to bringing the device to market and increasing acceptance of the device with patients and users.

## User needs can be either documented as a stand-alone document or integrated into the system-level product requirements document.  A template has been provided in TMP1-QM.SLQ006 as a recommended format for user needs / requirement document.  Sections are to be added, removed and/or adapted as necessary to ensure a well-organized and comprehensive set of user needs.

# Procedure:  Defining Design Inputs

## Based on the identified user needs, a set of design inputs (product requirements) that are clear, understandable, and testable are to be defined.  The output from this activity is a formalized set of design input documents.

## These documents can be either unified into a single integrated product requirement document with user needs, or can be a series of design input documents for different levels of the device (system, assembly, sub-assembly, peripherals, etc.)

## A template has been provided in TMP2-QM.SLQ006 as a recommended format for product requirement documents.  Sections are to be added, removed and/or adapted as necessary to ensure a well-organized and comprehensive set of design inputs.

## Product requirements are to be collected and documented from the following sources:

### Users (both patient and/or healthcare provider)

### Regulatory bodies

### Standards Organizations

### Internal design or functional groups

## To minimize requirement conflicts, design input documents should specify what the characteristics of the products are, not how the design will fulfill these requirements.  Note:  Although design inputs should not impart the detailed manifestation, design/development does not occur in a bubble and therefore design inputs are expected to be influenced by other design efforts.

## To define a comprehensive set of design inputs, the following elements are to be considered:

### User needs

### Clinical requirements

### Principle of operation of device

### Device functionality

### Safety requirements

### Device performance and reliability

### Electrical and power constraints

### Materials of construction

### Regulatory constraints

### Physical and ergonomic design constraints

### Biocompatibility

### Sterility

### Packaging and Labeling

### Manufacturability

### Installation and servicing constraints

### Existing similar devices, including historical concerns

## Design inputs are to be designated as a ‘must’ or ‘shall’ statement.  If a requirement is deemed optional an input can be stated as ‘should’ or ‘can’.  It is important to avoid stating a design input as ‘will’ or ‘is’ which typically is used with design outputs.

## Preliminary risk analysis activities are to be performed in conjunction with the design input process to identify product requirements that are necessary to minimize the severity and likelihood of potential risks/hazards to patients and users.  For more specific details see the Risk Analysis SOP, QM.SLQ013.

## It is expected that these documents will go through several iterations in finalizing the user needs and design inputs.  Design Input documents may be revised as necessary to address resolution of incomplete, ambiguous, or conflicting requirements.  In the event that incomplete, ambiguous, or conflicting requirements are discovered, SILQ management or their representatives will have final authority to make necessary changes to resolve such conflicts.  All revisions to design input documents are to be controlled through the DCO process.

## The culmination of product design and development results in the transformation of product design inputs into design outputs. These design outputs define the actual device design from a combination of product specifications and descriptive materials.

## Design verification activities are intended to confirm the device design meets specified design inputs.

## Design validation activities are intended to provide objective evidence that the final product conforms to established user needs and intended use.

# Procedure:  Traceability

## All identified design inputs / product requirements are to be assigned a unique identifier.  If a design input is removed, the assigned number is to be left in place and the requirement replaced with ‘Requirement obsoleted’ and the change order reference number provided for rationale of removal.

## To facilitate tracking and tracing of design activities between the processes of the design control program, a traceability matrix is to be created to trace design inputs by their identifier.

## The traceability matrix is to trace design inputs to design outputs and to their subsequent testing activities.

# Procedure:  Design History File (DHF)

## Approved user needs and design input documents are to be retained in the DHF for each specific product line.

### DHF documents are to be scanned and imported into FileHold and filed in the appropriate DHF.