# General Production Information

| Work Order #: |  | NCMR #: |  |
| --- | --- | --- | --- |
| Part Number: | C.SLQ001 | Part Revision: |  |
| Lot #: |  | Manufacture Date / Expiration Date: | Mfg date: Exp date: |
| Build Purpose: |  |  |  |  |
| Additional Build Info: |  |  |  |  |

# Material Traceability

| Part Number | Description | Part Revision | Lot / Batch # | Expiration Date | Qty. Obtained | Obtained By: (Initial/Date) |
| --- | --- | --- | --- | --- | --- | --- |
| M.SLQ001 | Polymeric Dispersion Solution, Unpurified, CHEMPSBTCI010 |  |  |  |  |  |

# Equipment Traceability

| Part Number | Description | Equipment ID | Calibration Due Date |
| --- | --- | --- | --- |
| E.SLQ006 | Diafiltration System, KrosFlo KTF-2000 |  |  |
| E.SLQ010 | Floor Scale, Concentrate Reactor |  |  |
| E.SLQ012 | Floor Scale, Suspension |  |  |

# Build Record

| Step | Procedure | Step Description | Status / Results | Performed By: (Initial/Date) |
| --- | --- | --- | --- | --- |
| 1 | QM.SLQ049 | Line clearance completed | Yes No N/A |  |
| 2 | MP-C.SLQ001 | Verify conductivity indicator on Portable Exchange Deionizer System (E.SLQ001) is green. | Indicator is green |  |
| 3 | MP-C.SLQ001 | Drain liquid in diafiltration system/tubing into waste tank. | Yes No |  |
| 4 | MP-C.SLQ001 | Connect Retentate Lines and Feed Lines to the appropriate pumps. Route lines through the appropriate membrane modules, flow meters, and backpressure valves. | Yes No |  |
| 5 | MP-C.SLQ001 | Place Permeate Line into Peristaltic Pump; change Valve 1 to ‘permeate line’ position. | Yes No |  |
| 6 | MP-C.SLQ001 | Tare Concentrate Reactor Floor Scale. Fill Concentrate Reactor with ~30kg of high purity water (S.SLQ001). | Floor scale tared Floor scale reading: _______________________ |  |
| 7 | MP-C.SLQ001 | ‘Feed (remaining)’ value on system software is within ±0.2kg of floor scale reading. | ‘Feed (Remaining)’ value: _______________________ Within ±0.2kg of floor scale reading |  |
| 8 | MP-C.SLQ001 | Manually run Pump 01 and Pump 02 until membrane modules and retentate lines are filled. | Yes No |  |
| 9 | MP-C.SLQ001 | Confirm no leaks are observed in all connections. | No leaks observed |  |
| 10 | MP-C.SLQ001 | Fill Concentrate Reactor until scale reads ~30kg. | Floor scale reading: _______________________ |  |
| 11 | MP-C.SLQ001 | Set parameters on main page of system software. - Starting Volume: 30L - DV Setpoint: 2.0 DV | Parameters set |  |
| 12 | MP-C.SLQ001 | Click START PROCESS button to fill membrane modules with water. After membrane modules are filled with water, ensure Permeate Valve is set to vertical position and open the four clamps on permeate line. | Membrane modules filled with water Permeate Valve set to vertical position Open four clamps on permeate line |  |
| 13 | MP-C.SLQ001 | Click Stop button when ‘Perm. Total’ value reads 60L on system software. | ‘Perm. Total’ value: ______________ |  |
| 14 | MP-C.SLQ001 | Mount Pneumatic Drum Stirrer into holder and turn on. | Yes No |  |
| 15 | MP-C.SLQ001 | Obtain container(s) of Unpurified Polymeric Dispersion Solution (M.SLQ001). Record traceability information in the Material Traceability table above. | Container(s) obtained Material traceability recorded |  |
| 16 | MP-C.SLQ001 | Record volume of container(s). | Volume of container(s): ____________________ |  |
| 17 | MP-C.SLQ001 | Fill Concentrate Reactor with high purity water until the Designated Fill Value (DFV) equals the volume of container(s). | Yes No Designated Fill Value (DFV): _____________________ |  |
| 18 | MP-C.SLQ001 | Calculate the Designated Operating Value (DOV). DOV = 10 / 3 * DFV. | Designated Operating Value (DOV): ______________________ |  |
| 19 | MP-C.SLQ001 | Slowly pour contents of Unpurified Polymeric Dispersion Solution container(s) into concentrate reactor. Ensure no gelation develops. | Yes No |  |
| 20 | MP-C.SLQ001 | Fill concentrate reactor with high purity water until DOV is reached (within ±2kg). Record floor scale and system software ‘Feed (Remaining)’ value. Verify values are within ±2kg of DOV and each other. | Floor scale reading: ________________ “Feed (Remaining)” value: ________________ Values are within ±2kg of DOV and each other |  |
| 21 | MP-C.SLQ001 | Set parameters on main page of system software. - Starting Volume: Designated Operating Value - DV Setpoint: 10 DV | Parameters set |  |
| 22 | MP-C.SLQ001 | Click START PROCESS button. Verify Pump 01 and Pump 02 are running | Pump 01 and Pump 02 running |  |
| 23 | MP-C.SLQ001 | Run process until “Perm. Total” value reaches 10 times the value of the Designated Fill Value (see Step 17 above). | “Perm. Total” value: _____________________ |  |
| 24 | MP-C.SLQ001 | Record weight of product tank (C.SLQ003) without the lid. | Product tank weight: _____________________ kg |  |
| 25 | MP-C.SLQ001 | Transfer Suspension to plastic drum using the peristaltic pump via the product line (C.SLQ003). | Yes No |  |
| 26 | MP-C.SLQ001 | Take sample of suspension in 50mL tubes at the beginning, middle, and end of the transfer. Label each tube Aliquot A, Aliquot B, and Aliquot C, respectively. | Yes No |  |
| 27 | MP-C.SLQ001 | Place clean containers below Pump 01 and Pump 02. Disconnect retentate lines from pumps and allow suspension to drain into containers. Pour contents of the containers into the product tank. | Yes No |  |
| 28 | MP-C.SLQ001 | Record the initial weight of the suspension. Assumption: The initial weight (in kg) of the suspension is equal to the weight of product tank subtracted from the weight of the suspension and product tank. | Initial Suspension Weight: ___________________ kg |  |
| 29 | MP-C.SLQ001 | Seal product tank. | Yes No |  |
| 30 | MP-C.SLQ001 | Create product label for Suspension (L.SLQ005) indicating lot number and expiration date of suspension. Apply label to product tank. | Yes No |  |
| 31 | MP-C.SLQ001 | Place product tank in quarantine pending QC analysis of suspension. | Yes No |  |
| 32 | MP-C.SLQ001 | Prepare KTF-2000 System for storage Fill concentrate reactor with ~30kg of high purity water and manually start running Pump 01 and Pump 02. Allow Membrane Modules 1 and 2 and Retentate Lines 1A, 1B, 2A, and 2B are filled. Allow pumps to run for an additional 10 minutes. | Yes No |  |
| 33 | MP-C.SLQ001 | Switch Permeate Line Valve to horizontal (closed) position and Waste Line Valve to vertical (open) position. Place Waste Line through peristaltic pump and into the concentrate reactor. Manually run the peristaltic pump to fully drain the contents of the concentrate reactor into the waste tank. | Yes No |  |
| 34 | MP-C.SLQ001 | Create Storage Solution. Combine 20L of high purity water + 1.5mL of bleach solution (S.SLQ007). | Yes No |  |
| 35 | MP-C.SLQ001 | Pour Storage Solution into Concentrate Reactor. Close 4 Permeate Line Clamps and switch Permeate Line Valve to horizontal (closed) position and Waste Line Valve to vertical (open) position. | Yes No |  |
| 36 | MP-C.SLQ001 | Manually run Pump 01 and Pump 02 until Membrane Modules 1 and 2 and Retentate Lines 1A, 1B, 2A, and 2B are filled. | Yes No |  |
| 37 | MP-C.SLQ001 | Once filled, stop Pump 01 and Pump 02 and close the ball valves for Retentate Lines 1B and 2B at the Concentrate Reactor. Allow 5 minutes to elapse for settling and degassing. Ensure the Storage Solution has filled Membrane Modules 1 and 2 entirely. Storage Solution should at least fill Retentate Lines 1B and 2B up to the respective line’s Flow Meter. | Yes No |  |
| 38 | MP-C.SLQ001 | Wait 5 minutes to ensure the Membrane Modules are full and stable. | Yes No |  |
| 39 | MP-C.SLQ001 | Manually run the Peristaltic Pump to fully drain the contents of the Concentrate Reactor into the Waste Tank. | Yes No |  |
| 40 | MP-C.SLQ001 | Power down Pump 03, Pump 04, concentrate reactor scale, suspension scale, and KTF system software. | Yes No |  |
| 41 | MP-C.SLQ001 | Prepare the high purity water tank for storage Take a peristaltic pump with sterilized tubing to pump out any residual high purity water from the tank. | Yes No |  |
| 42 | MP-C.SLQ001 | Allow high purity tank to dry for 24 hours before securing lid to the tank. | Yes No |  |
| 43 | MP-C.SLQ001 | Clean containers using in the manufacturing process. Allow containers to dry and store after they have dried. | Yes No |  |
| 44 | QC-C.SLQ001 | Execution of QC analysis of suspension samples. QC analysis report attached to DHR. | Yes No |  |
| 45 | MP-C.SLQ001 | Attach Suspension Quality Control Report and COA to DHR. | Yes No |  |
| 46 | MP-C.SLQ001 | Test results meet acceptance criteria. | Yes No |  |
| 47 | MP-C.SLQ001 | Dilution Procedure Calculate the Average Suspension Sample Concentration (g/L) by taking the average ‘Residue Net Weight’ of six (6) freeze dried suspension samples based off the 10 mL sample. | Average Suspension Sample Concentration: __________________ g/L |  |
| 48 | MP-C.SLQ001 | Determine if Concentration Process is required. If Average Suspension Sample Concentration is <10g/L, Concentrate Process is required. If Average Suspension Sample Concentration is >10g/L, Dilution Procedure can proceed. | Is Concentration Process Required? Yes, skip Steps 49-54 and proceed to Step A1. No, proceed to Step 49. |  |
| 49 | MP-C.SLQ001 | Calculate the volume (L) of high purity water to be added to the product tank. Volume = [Average Suspension Sample Concentration (g/L) * Initial Suspension Volume (L)]/[10 (g/L)] – Initial Suspension Volume (L) | Volume of high purity water to be added to product tank: __________________ L |  |
| 50 | MP-C.SLQ001 | Tare suspension floor scale and place product tank onto floor scale. | Yes No |  |
| 51 | MP-C.SLQ001 | Add volume of high purity water (from Step 35) to the product tank. | Yes No |  |
| 52 | MP-C.SLQ001 | Record the weight of product tank with suspension. | _____________________ kg |  |
| 53 | MP-C.SLQ001 | Calculate weight of suspension. | ______________________ kg |  |
| 54 | MP-C.SLQ001 | Update product label with weight of suspension. | Yes No |  |

Comments/Additional Notes:

Build Record Suspension Concentration Process

(Check one:   Required,      Not Required)

| Step | Procedure | Step Description | Status / Results | Performed By: (Initial/Date) |
| --- | --- | --- | --- | --- |
| A1 | QM.SLQ049 | Line clearance completed. | Yes No N/A |  |
| A2 | MP-C.SLQ001 | Drain liquid in diafiltration system/tubing into waste tank. | Yes No |  |
| A3 | MP-C.SLQ001 | Connect Retantate Lines and Feed Lines to the appropriate pumps. Route lines through the appropriate membrane modules, flow meters, and backpressure valves. | Yes No |  |
| A4 | MP-C.SLQ001 | Place Permeate Line into Peristaltic Pump; change Valve 1 to ‘permeate line’ position. | Yes No |  |
| A5 | MP-C.SLQ001 | Tare Concentrate Reactor Floor Scale. Fill Concentrate Reactor with ~30kg of high purity water (S.SLQ001). | Floor scale tared Floor scale reading: _______________________ |  |
| A6 | MP-C.SLQ001 | ‘Feed (remaining)’ value on system software is within ±0.2kg of floor scale reading. | ‘Feed (Remaining)’ value: _______________________ Within ±0.2kg of floor scale reading |  |
| A7 | MP-C.SLQ001 | Manually run Pump 01 and Pump 02 until membrane modules and retentate lines are filled. | Yes No |  |
| A8 | MP-C.SLQ001 | Confirm no leaks are observed in all connections. | No leaks observed |  |
| A9 | MP-C.SLQ001 | Fill Concentrate Reactor until scale reads ~30kg. | Floor scale reading: _______________________ |  |
| A10 | MP-C.SLQ001 | Set parameters on main page of system software. - Starting Volume: 30L - DV Setpoint: 2.0 DV | Parameters set |  |
| A11 | MP-C.SLQ001 | Click START PROCESS button to fill membrane modules with water. After membrane modules are filled with water, ensure Permeate Valve is set to vertical position and open the four clamps on permeate line. | Membrane modules filled with water Permeate Valve set to vertical position Open four clamps on permeate line |  |
| A12 | MP-C.SLQ001 | Click Stop button when ‘Perm. Total’ value reads 60L on system software. | ‘Perm. Total’ value: ______________ |  |
| A13 | MP-C.SLQ001 | Mount Pneumatic Drum Stirrer into holder and turn on. | Yes No |  |
| A14 | MP-C.SLQ001 | Drain liquid in diafiltration system/tubing into waste tank. | Yes No |  |
| A15 | MP-C.SLQ001 | Calculate Concentration Permeate Required Value: Concentration Permeate Required = [12 (g/L)-Average Suspension Sample Concentration (g/L)]/ 12 (g/L) * Initial Suspension Volume (L) + 16 (L) | Concentration Permeate Required (L): |  |
| A16 | MP-C.SLQ001 | Tare Concentrate Reactor Floor Scale. | Floor scale tared |  |
| A17 | MP-C.SLQ001 | Transfer Suspension from plastic drum using the peristaltic pump via the product line (C.SLQ003) into Concentrate Reactor. | Yes No |  |
| A18 | MP-C.SLQ001 | Manually run Pump 01 and Pump 02 until “Perm. Total” value reaches Concentration Permeate Required Value. Note: No High Purity Water will be added to Concentrate Reactor and concentrate Reactor Floor Scale reading will gradually decrease. | “Perm. Total” value: _____________________ |  |
| A19 | MP-C.SLQ001 | Tare suspension floor scale and place product tank onto floor scale. | Yes No |  |
| A20 | MP-C.SLQ001 | Record weight of product tank (C.SLQ003) without the lid. | Product tank weight: _____________________ kg |  |
| A21 | MP-C.SLQ001 | Transfer Suspension to plastic drum using the peristaltic pump via the product line (C.SLQ003). | Yes No |  |
| A22 | MP-C.SLQ001 | Take sample of suspension in 50mL tubes at the beginning, middle, and end of the transfer. Label each tube Aliquot A, Aliquot B, and Aliquot C, respectively. | Yes No |  |
| A23 | MP-C.SLQ001 | Place clean containers below Pump 01 and Pump 02. Disconnect retentate lines from pumps and allow suspension to drain into containers. Pour contents of the containers into the product tank. | Yes No |  |
| A24 | MP-C.SLQ001 | Record the initial weight of the suspension. Assumption: The initial weight (in kg) of the suspension is equal to the weight of product tank subtracted from the weight of the suspension and product tank. | Initial Suspension Weight: ___________________ kg |  |
| A25 | MP-C.SLQ001 | Seal product tank. | Yes No |  |
| A26 | MP-C.SLQ001 | Create product label for Suspension (L.SLQ005) indicating lot number and expiration date of suspension. Apply label to product tank. | Yes No |  |
| A27 | MP-C.SLQ001 | Place product tank in quarantine pending QC analysis of suspension. | Yes No |  |
| A28 | MP-C.SLQ001 | Prepare KTF-2000 System for storage Fill concentrate reactor with ~30kg of high purity water and manually start running Pump 01 and Pump 02. Allow Membrane Modules 1 and 2 and Retentate Lines 1A, 1B, 2A, and 2B are filled. Allow pumps to run for an additional 10 minutes. | Yes No |  |
| A29 | MP-C.SLQ001 | Switch Permeate Line Valve to horizontal (closed) position and Waste Line Valve to vertical (open) position. Place Waste Line through peristaltic pump and into the concentrate reactor. Manually run the peristaltic pump to fully drain the contents of the concentrate reactor into the waste tank. | Yes No |  |
| A30 | MP-C.SLQ001 | Create Storage Solution. Combine 20L of high purity water + 1.5mL of bleach solution (S.SLQ007). | Yes No |  |
| A31 | MP-C.SLQ001 | Pour Storage Solution into Concentrate Reactor. Close 4 Permeate Line Clamps and switch Permeate Line Valve to horizontal (closed) position and Waste Line Valve to vertical (open) position. | Yes No |  |
| A32 | MP-C.SLQ001 | Manually run Pump 01 and Pump 02 until Membrane Modules 1 and 2 and Retentate Lines 1A, 1B, 2A, and 2B are filled. | Yes No |  |
| A33 | MP-C.SLQ001 | Once filled, stop Pump 01 and Pump 02 and close the ball valves for Retentate Lines 1B and 2B at the Concentrate Reactor. Allow 5 minutes to elapse for settling and degassing. Ensure the Storage Solution has filled Membrane Modules 1 and 2 entirely. Storage Solution should at least fill Retentate Lines 1B and 2B up to the respective line’s Flow Meter. | Yes No |  |
| A34 | MP-C.SLQ001 | Wait 5 minutes to ensure the Membrane Modules are full and stable. | Yes No |  |
| A35 | MP-C.SLQ001 | Manually run the Peristaltic Pump to fully drain the contents of the Concentrate Reactor into the Waste Tank. | Yes No |  |
| A36 | MP-C.SLQ001 | Power down Pump 03, Pump 04, concentrate reactor scale, suspension scale, and KTF system software. | Yes No |  |
| A37 | MP-C.SLQ001 | Prepare the high purity water tank for storage Take a peristaltic pump with sterilized tubing to pump out any residual high purity water from the tank. | Yes No |  |
| A38 | MP-C.SLQ001 | Allow high purity tank to dry for 24 hours before securing lid to the tank. | Yes No |  |
| A39 | MP-C.SLQ001 | Clean containers using in the manufacturing process. Allow containers to dry and store after they have dried. | Yes No |  |
| A40 | QC-C.SLQ001 | Execution of QC analysis of suspension samples. QC analysis report attached to DHR. | Yes No |  |
| A41 | MP-C.SLQ001 | Attach Suspension Quality Control Report and COA to DHR. | Yes No |  |
| A42 | MP-C.SLQ001 | Test results meet acceptance criteria. | Yes No |  |
| A43 | MP-C.SLQ001 | Dilution Procedure Calculate the Average Suspension Sample Concentration (g/L) by taking the average ‘Residue Net Weight’ of six (6) freeze dried suspension samples based off the 10 mL sample. | Average Suspension Sample Concentration: __________________ g/L |  |
| A44 | MP-C.SLQ001 | Determine if Concentration Process is required. If Average Suspension Sample Concentration is <10g/L, Concentrate Process is required again. If Average Suspension Sample Concentration is >10g/L, Dilution Procedure can proceed. | Is Concentration Process Required? Yes, skip Steps A45-A50 and proceed to Step A1 and repeat. No, proceed to Step A45. |  |
| A45 | MP-C.SLQ001 | Calculate the volume (L) of high purity water to be added to the product tank. Volume = [Average Suspension Sample Concentration (g/L) * Initial Suspension Volume (L)]/[10 (g/L)] – Initial Suspension Volume (L) | Volume of high purity water to be added to product tank: __________________ L |  |
| A46 | MP-C.SLQ001 | Tare suspension floor scale and place product tank onto floor scale. | Yes No |  |
| A47 | MP-C.SLQ001 | Add volume of high purity water (from Step 35) to the product tank. | Yes No |  |
| A48 | MP-C.SLQ001 | Record the weight of product tank with suspension | _____________________ kg |  |
| A49 | MP-C.SLQ001 | Calculate weight of suspension. | ______________________ kg |  |
| A50 | MP-C.SLQ001 | Update product label with weight of suspension. | Yes No |  |

End of procedure

Comments/Additional Notes: